# ENGINEERING PROPOSAL TEMPLATE

# Solar Street Light Project

Tender-ready editable MS Word template for EPC contractors and procurement teams.

### Project: [PROJECT_NAME]

### Client: [CLIENT_NAME]

### Location: [COUNTRY_CITY]

### Date: [DATE]

Prepared by: SUNLURIO (sunlurio.com) | info@sunlurio.com

How to use: Replace all placeholders in [BRACKETS]. Update the Table of Contents by right-clicking it and selecting 'Update Field'. Attach the simulation report and IES files as appendices.

# Table of Contents

Tip: In Word, click inside the table above and press F9 (or right-click > Update Field) after you finish editing.

# 1. Executive Summary

This proposal presents a solar street lighting solution for [PROJECT_NAME] in [COUNTRY_CITY]. The system is designed to meet [TARGET_STANDARD] Class [CLASS] (or client specification), verified by a lighting simulation report based on the site inputs provided.

Project coverage: [ROAD_LENGTH] / [AREAS] | Estimated quantity: [QTY] sets

Key risk controls: uniformity between poles, rainy-season autonomy, acceptance checklist and handover documents

Deliverables: BOQ, technical specifications, installation method statement, QA/QC plan, and acceptance checklist

### Decision-ready summary (fill in):

| Item | Value |
| --- | --- |
| Road width / pole height / spacing | [ROAD_WIDTH_M] m / [POLE_HEIGHT_M] m / [POLE_SPACING_M] m |
| Target performance | Eavg [EAVG_LUX] lux ¦ U0 [U0] ¦ Emin [EMIN_LUX] lux (if required) |
| Autonomy & operation | [AUTONOMY_NIGHTS] nights ¦ Dimming profile: [DIMMING_PROFILE] |
| Environment | [ENVIRONMENT] (e.g., coastal/dusty/humid) ¦ Max temp [TMAX] C |
| Delivery terms | [DELIVERY_TERM] ¦ Port: [PORT] ¦ Lead time: [LEAD_TIME_WEEKS] weeks |

# 2. Project Scope and Assumptions

## 2.1 Scope of Supply

Solar street light system (LED luminaire + solar panel + LiFePO4 battery + MPPT controller)

Mounting structure (pole/arm) and anchor bolt kit (if included in this offer)

Cables, connectors, fasteners, anti-theft accessories (as specified)

Documentation pack (datasheets, wiring diagram, installation guide, warranty)

## 2.2 Exclusions (tick as applicable)

☐ Civil works beyond agreed foundation scope

☐ Special permits, local taxes, and statutory fees

☐ Grid connection (for hybrid systems) unless specified

## 2.3 Assumptions

Site inputs listed in Section 3 are accurate and approved by the client/EPC.

Lighting targets follow [TARGET_STANDARD] or client specification. If targets are not provided, supplier will recommend targets based on road type and usage.

Battery autonomy is sized for [AUTONOMY_NIGHTS] nights under the dimming profile stated.

# 3. Site Inputs (Required for Simulation)

Provide these inputs to ensure the simulation and BOQ are defensible for tender submission.

| Category | Parameter | Value (fill in) |
| --- | --- | --- |
| Geometry (Required) | Road width (m) | [ROAD_WIDTH_M] |
| Geometry (Required) | Pole height (m) | [POLE_HEIGHT_M] |
| Geometry (Required) | Pole spacing (m) | [POLE_SPACING_M] |
| Layout (Recommended) | Arrangement (one-side/both-side/staggered/median) | [ARRANGEMENT] |
| Layout (Recommended) | Setback to curb (m) / Arm length (m) / Tilt angle (deg) | [SETBACK_M] / [ARM_M] / [TILT_DEG] |
| Targets | Eavg (lux) / U0 / Emin (lux) (if required) | [EAVG_LUX] / [U0] / [EMIN_LUX] |
| Solar Operation | Autonomy nights / Dimming profile / Motion sensor | [AUTONOMY_NIGHTS] / [DIMMING_PROFILE] / [SENSOR_YN] |
| Environment | Coastal/dusty/humid/normal ¦ Typical/Max temperature (C) | [ENVIRONMENT] ¦ [TTYP] / [TMAX] |

Attachments (recommended): site layout DWG/PDF, Google Maps pin/coordinates, and site photos.

# 4. Lighting Design Approach and Standards

## 4.1 Photometric Data and File Formats

Photometric files define how light leaves the luminaire. Road illuminance and uniformity depend on optics plus geometry (height/spacing/aiming) and the selected lighting criteria.

IES file (LM-63) provided for the selected optic; LDT available on request.

Simulation software: DIALux evo / Relux (as agreed).

Luminaire verification: LM-79 photometric report available on request.

## 4.2 Standards and Acceptance Targets

Reference standard: [TARGET_STANDARD] (e.g., EN 13201 / IES RP-8 / Client Spec).

Targets: Eavg [EAVG_LUX] lux, Uniformity U0 [U0], and any additional requirements (glare/illuminance class).

Maintenance Factor (MF): [MF] (final MF to follow environment and O&M plan).

## 4.3 Simulation Deliverables (to attach)

Lighting Simulation Report (PDF): isolux map + calculation summary table (Eavg, U0, Emin).

Layout notes: recommended spacing/height/arm adjustments to eliminate dark zones between poles.

Option A/B (optional): cost-optimized vs rainy-season optimized design.

# 5. Technical Specifications

## 5.1 System Overview

All specifications below are project-dependent and should be finalized after simulation and site confirmation.

| Component | Specification (fill in) | Notes |
| --- | --- | --- |
| Solar panel | [PANEL_W] W ¦ [PANEL_TYPE] ¦ Bracket: [BRACKET_TYPE] | Sized by location and autonomy target |
| Battery | LiFePO4 ¦ [BATTERY_V] V ¦ [BATTERY_WH] Wh ¦ [CYCLES] cycles | Capacity based on autonomy nights and dimming profile |
| Controller | MPPT ¦ Protections: OVP/UVP/OTP ¦ Dimming: [DIMMING_MODE] | Remote control optional: 4G/5G/NB-IoT (if required) |
| LED luminaire | [LED_W] W ¦ Optic: [OPTIC_TYPE] ¦ Efficacy: [SYSTEM_LM_W] lm/W | Optics drive uniformity between poles |
| CCT / CRI | [CCT] K ¦ CRI [CRI] | Specify binning if required (e.g., 3-step MacAdam) |
| Ingress protection | IP[IP] ¦ IK[IK] | Outdoor reliability |
| Surge protection | SPD: [SPD_KV] kV | Recommended in lightning regions |

## 5.2 Pole and Foundation (if included)

| Item | Value (fill in) | Notes |
| --- | --- | --- |
| Pole height | [POLE_HEIGHT_M] m | Match simulation layout |
| Material & finish | Hot-dip galvanized steel ¦ coating: [COATING] | Specify corrosion class for coastal sites |
| Base plate / bolt circle | [BOLT_CIRCLE_MM] mm ¦ holes: [HOLES] | Provide drawing and template |
| Anchor bolts | M[ANCHOR_M] x [ANCHOR_LEN] mm | Request casting template before pouring concrete |
| Bolt projection length | [BOLT_PROJ_MM] mm above foundation | Critical for nut engagement and leveling range |

# 6. Bill of Quantities (BOQ) and Budget

## 6.1 BOQ Table

| Item | Unit | Qty | Notes |
| --- | --- | --- | --- |
| Solar street light set (complete) | set | [QTY] | Panel + LiFePO4 battery + controller + luminaire |
| Pole + arm (if included) | set | [QTY] | [POLE_HEIGHT_M] m |
| Anchor bolt kit (if included) | set | [QTY] | With template + projection length |
| Installation & commissioning | job | 1 | As per scope |
| Spares kit (recommended) | lot | 1 | 2-3% critical parts |

## 6.2 Budget Assumptions

Pricing depends on autonomy nights, dimming profile, optics selection, pole/foundation scope, and delivery terms.

State what is included/excluded for civil works, taxes, and local permits.

Provide Good/Better/Best options if requested.

# 7. Packaging and Logistics

Delivery term: [DELIVERY_TERM] | Destination port: [PORT]

Packing list and shipping marks provided with each shipment.

Documents: Commercial invoice, packing list, bill of lading/airway bill, certificate of origin (if required).

# 8. Installation Method Statement

## 8.1 Pre-installation Checks

Verify anchor bolt template and bolt projection length before concrete pour.

Inspect all components on arrival (panel, luminaire, battery, controller, pole).

Confirm layout matches the approved simulation (height, spacing, arrangement, tilt).

## 8.2 Installation Steps (Typical)

Foundation and anchor bolt installation (per approved drawing).

Pole erection and leveling; torque tightening to specified value.

Mount luminaire, arm, panel bracket; set tilt angle to [TILT_DEG] degrees (or approved).

Cable routing and waterproof connection; verify polarity and grounding.

Controller configuration: dimming profile [DIMMING_PROFILE] and sensor settings (if any).

## 8.3 Commissioning

Daytime check: charging status and PV input.

Night check: light output, dimming schedule, and sensor function (if applicable).

Record settings and serial numbers for as-built documentation.

# 9. QA/QC Plan

## 9.1 Factory Acceptance (FAT) - Examples

Incoming inspection for key components (cells, controller, LED modules).

Functional test and burn-in.

Waterproof sealing inspection.

CCT consistency check (avoid mixed appearance on the road).

Packaging inspection and serial tracking.

## 9.2 Site Acceptance (SAT) - Examples

Mechanical inspection: mounting, torque, cable routing, sealing.

Configuration verification: dimming schedule and sensor settings.

Night operation verification.

Lux sampling at agreed points (if required by consultant).

# 10. Acceptance and Handover Checklist

Use this checklist to ensure smooth handover and payment release.

| Checkpoint | Pass | Notes / Record |
| --- | --- | --- |
| Anchor bolts match base plate; bolt projection length verified | ☐ |  |
| Pole plumb and leveled; fasteners torqued | ☐ |  |
| Panel orientation and tilt set; no shading | ☐ |  |
| Waterproof connectors and cable routing completed | ☐ |  |
| Controller settings recorded (dimming profile, sensor) | ☐ |  |
| Night test passed (output + dimming schedule) | ☐ |  |
| CCT consistency acceptable (no zebra effect) | ☐ |  |
| As-built serial list completed | ☐ |  |

## 10.2 Handover Documents

Final BOQ and packing list

Wiring diagram and installation manual

Warranty certificate

As-built: serial numbers and locations

Simulation report and IES files (appendix)

# 11. Warranty and After-sales Support

| Item | Warranty (fill in) | Conditions / Notes |
| --- | --- | --- |
| System | [WARRANTY_YEARS] years | As per warranty policy |
| Battery (LiFePO4) | [BATTERY_WARRANTY_YEARS] years | Based on operation profile and protections |
| LED luminaire | [LUM_WARRANTY_YEARS] years | CCT binning and driver conditions as specified |

Recommended spares: 2-3% critical parts for project continuity.

# 12. Appendix (Attach as Needed)

Appendix A - Lighting Simulation Report (PDF)

Appendix B - IES (LM-63) files for selected optics (and LDT on request)

Appendix C - Datasheets and drawings (pole, base plate, anchor bolt template)

Appendix D - Certificates and test reports (LM-79 on request)

Appendix E - Packing list sample and shipping documents (as required)