Tender/RFP Photometry Request Language Pack (IES/LDT)

Copy-paste ready clauses for EPC tenders and submittals

Version: 2026-01-20

1. Definitions

Photometric File(s): Editable electronic photometric data files in IES (.ies) and/or EULUMDAT (.ldt) format used for lighting calculation and simulation.

Bid Variant: A unique configuration identified by catalog/SKU, optic/distribution, wattage/drive current (or power setting), CCT, CRI, and mounting orientation.

2. Mandatory Photometry Deliverables

2.1 Editable Raw Files (No PDF-only). Bidder shall submit IES and/or LDT photometric file(s) as editable text-based files. Polar plots or PDFs are supplementary only and shall not substitute the editable photometric file(s).

2.2 Variant-Correct Photometry. Photometric file(s) shall correspond to the exact Bid Variant offered. Generic family/series photometry is not acceptable unless a Variant Mapping Table and scaling justification are provided (see Section 5).

2.3 File Naming Convention. All photometric files shall follow naming: [ProjectCode]_[FixtureCode]_[Optic]_[Power/Watt]_[CCT]_[Rev]_[TestDate].ies/.ldt (example: KGLA_Lot1_Area_Wide_120W_4000K_R02_2025-10-18.ies).

3. Test Traceability and Measurement Basis

3.1 Test Method Reference. Bidder shall provide a Test Summary referencing ANSI/IES LM-79 and/or CIE S 025 (or equivalent) for the Bid Variant photometry and electrical measurements.

3.2 Minimum Test Summary Content (mandatory): (a) test laboratory name and address; (b) test standard reference; (c) test date and report reference/ID (if available); (d) measured input power (W), total luminous flux (lm), CCT, CRI; (e) photometry type confirmation: absolute vs relative, with clear scaling instruction (Section 4).

4. Absolute vs Relative Photometry Control

4.1 Declaration Required. Bidder shall explicitly declare whether photometry is absolute (measured luminaire output) or relative (normalized) and provide the correct simulation scaling instruction: scale once / no scaling.

4.2 No Double-Scaling. If the photometry is relative, bidder shall provide the exact lumen basis and multiplier rules, and EPC shall apply scaling exactly once.

5. Variant Mapping Table Requirement

5.1 Mandatory for Multi-Variant / Family Offers. Where multiple optics, wattages, drive currents, CCT/CRI options, or mechanical variants exist, bidder shall provide a completed Variant Mapping Table linking each Bid Variant to its photometric file name, test date, and revision.

5.2 Reject Without Mapping. Failure to submit the Variant Mapping Table for a family/multi-variant offer shall be treated as non-responsive.

6. Laboratory Competence Requirement

6.1 ISO/IEC 17025 (Preferred/Required). Photometric and electrical test reports shall be issued by a laboratory accredited to ISO/IEC 17025 (or equivalent national accreditation scheme). Reports must be issued within the laboratory's accreditation effective period and scope.

7. Revision Control and Change Notification

7.1 Revision Control. Photometric files shall include a revision identifier. Any change affecting optical output (optics, lens, LED package, drive current, housing geometry, aiming) shall require a new photometric file revision and written change note.

7.2 Substitution Prohibited. After award, bidder shall not substitute photometric files or product variants without Employer approval and updated mapping/test summary.

8. File Authenticity / No Re-export as Substitute

8.1 Raw Photometry. Bidder shall submit photometric files generated from laboratory test output. Files exported/re-generated from lighting design software shall not be used as a substitute for the original tested photometry submission. Software exports may be submitted only as supplemental.

9. Right to Verify / Reject

Employer reserves the right to perform file sanity checks, import checks, and reference-scene simulations. If photometric files are missing key identifiers, show inconsistent variant labeling, or conflict with submitted Test Summary, Employer may request resubmission or reject the offer.

Note: Replace bracketed placeholders (e.g., [ProjectCode]) with your project identifiers. Keep the final file revisions used for modeling archived with your submittal package.